SRC MACHINE

Details of the SRC Machine are as follows;

Basic Material Specifications

Wet laid nonwovens + others

Thickness ranges from 350gsm – 450gsm

Minimum product tensile 100N/m

Unwind Section

Max web width 1800mm

Max rewind/unwind weight 250/300kgs

Max unwind roll diameter 1200mm

Braked unwind with load cell tension control

Automatic edge guider (+/- 50mm)

Reel lift arms on the unwind

Shaft-less unwind unit (min web width 400mm – max web width 1800mm)

Web splice and inspection table

Unwind cores i/d 76mm and 152mm chucking facilities

Nip Section

Driven draw rollers, with pneumatically actuated nip roller

Slitting Section

Rotary shear slitting (tangential)

16 individual shaft mounted female knives

16  individual pneumatic dovetail mounted shear slitting knives

Minimum slitting width 50mm

Normally slitting 300-400mm widths

Rewind Section

Simplex centre-surface rewind

Maximum rewind diameter 1200mm

Rewind mode – locked core (airshaft) – 1 off rewind airshaft 76mm

Off-loading the rewind support arms will lower to floor (similar to the Parkland system)

Rewind roll removal by pump truck with bogey arrangement as currently in use on the parkland system (not part of selling scope)

Two drive machine – one draw roller drive and one rewind roller drive

Machine speed 200mpm (maximum mechanical speed).

THIS INFORMATION IS THE TITLE OF JOHN WILKIE PAPER MILL SERVICES LTD AND SHOULD NOT BE COPIED OR FORWARDED TO A THIRD PARTY WITHOUT CONSENT.

FOR YOUR GENERAL INFORMATION WE HAVE TO INTIMATE THE FOLLOWING TO YOU:

This schedule has been prepared solely for the guidance of prospective purchasers and does not constitute (and at no time shall be deemed to constitute) an offer or contract or any part of an offer or contract. Additionally, no warranty is given to the accuracy of any measurements, specifications or other details whatsoever given within this schedule.

Whilst every reasonable effort has been made to verify any statement, description, or comment made within this schedule, for the avoidance of doubt it is emphasised that the purchaser must make their own enquiries and satisfy themselves.